Characterization of Fine-Grained Associativity Gaps 

A Preliminary Study for X-Analysis Integration (XAI) 
and Other Engineering Information Problems

GIT Technical Report EL004-2002A

http://eislab.gatech.edu/pubs/reports/EL004/
October 29, 2002 - DRAFT

Russell S. Peak

Senior Researcher

Manufacturing Research Center

http://www.marc.gatech.edu/
Georgia Institute of Technology

Atlanta, Georgia USA


[image: image1.png]



Abstract

This report describes an initial study towards characterizing the information associativity gap and other engineering interoperability problems.  Drawing on over a decade of X-analysis integration (XAI)
 research and development, it uses the XAI multi-representation architecture (MRA) as a means to decompose the problem and guide identification of potential key metrics. 

A few such metrics are highlighted from the aerospace industry.  These include number of structural analysis users, number of analysis templates, and identification of computing environment components (e.g., number of CAD and CAE tools used in an example aerospace electronics design environment).

One problem, denoted the fine-grained associativity gap, is highlighted in particular.  Today such a gap in the CAD-CAE arena typically requires manual effort to connect an attribute from a design model (CAD) with attributes in one of its analysis models (CAE).  This report estimates that 1 million such gaps exist in the structural analysis a complex product like an airframe.  The labor cost alone to manually maintain such gaps likely runs in the tens of millions of dollars.  Other associativity gap costs have yet to be estimated, including over- and under-design, lack of knowledge capture, and inconsistencies.

Narrowing in on fundamental gaps like fine-grained associativity helps both to characterize the cost of today’s problems and to identify basic solution needs.  Other studies are recommended to explore such facets further.

Document Citation:

R. S. Peak (October 29, 2002 - DRAFT) Characterization of Fine-Grained Associativity Gaps: A Preliminary Study for X-Analysis Integration (XAI) and Other Engineering Information Problems. Georgia Tech Manufacturing Research Center.  Technical Report EL004-2002A.  

Available at: http://eislab.gatech.edu/pubs/reports/EL004/
Nomenclature

ABB
analysis building block

API
application programming interface

APM
analyzable product model

BGA
ball grid array

CAD
computer-aided design

CAE
computer-aided engineering (physics-based analysis in this context)

CBAM
context-based analysis model

COB
constrained object

CORBA
common object request broker (ORB) architecture

DR&O
design requirements and objectives

FEA
finite element analysis

KBE
knowledge-based engineering

MRA
multi-representation architecture

M&S
modeling and simulation

PDM
product data management

SMM
solution method model

SBE
simulation-based engineering

SOAP
simple object access protocol

1 Background:  Recent XAI Progress

	[image: image2.wmf]1   Solution Method Model

Y

ABB

SMM

2   Analysis Building Block

4   Context-Based Analysis Model

3

SMM

ABB

F

APM

ABB

CBAM

APM

Design Tools

Solution Tools

Printed Wiring Assembly (PWA)

Solder Joint

Component

PWB

body

3

body

2

body

1

body

4

T

0

Printed Wiring Board (PWB)

Solder

Joint

Component

Analyzable

Product Model


	[image: image3.wmf]Subsystem-S

Object Relationship Diagram-S

COB Structure

Definition Language

(

COS

)

I/O Table-S

Constraint Graph-S

Constraint Schematic-S

STEP

Express

Express-G




Figure 1 - Multi-representation architecture (MRA) and constrained object (COB) representation for analysis templates [1]

First we overview a framework developed over the past decade to address CAD-CAE interoperability needs.  This framework also helps us more precisely identify basic issues and metrics.

The multi-representation architecture (MRA) in Figure 1 is the conceptual foundation of an X-analysis integration (XAI) methodology based on object-oriented patterns that naturally exist in engineering analysis processes [1].  It is particularly aimed at design-analysis integration in CAD/CAE environments with high diversity (e.g., diversity of parts, analysis discipline, analysis idealization fidelity, design tools, and analysis tools) and where explicit design-analysis associativity is important (e.g., for automation, knowledge capture, and auditing).  In this context, analysis means simulating the physical behavior of a part or system (e.g., determining the stress in a circuit board solder joint).

The constrained object (COB) knowledge representation captures engineering semantics in a modular, reusable manner due to its object-oriented non-causal nature.  COBs support the MRA to address the specific needs of engineering analysis integration for simulation-based engineering (SBE), including virtual prototyping, KBE, and CAD-CAE interoperability.  

Analysis integration applications of these capabilities include support for design synthesis (sizing) and design verification (analysis), and implementation of MRA concepts as four main types of objects: 

· Analyzable product models (APMs): Represent knowledge-based design models augmented with analysis-oriented overlays. Include multi-fidelity idealizations and multi-source design information coordination.

· Context-based analysis models (CBAMs): Represent product-specific analysis modules/templates. Capture idealization decisions inside CAD-CAE associativity relations. Connect APMs and ABBs.

· Analysis building blocks (ABBs): Represent product-independent analytical concepts as reusable, modular, tool-independent objects. Generate SMMs for solution purposes.

· Solution method modules (SMMs): Represent solution method-specific models. Support white box reuse of existing tools (e.g., FEA tools and in-house codes).  Automatic interactions occur through native command lines or APIs, and/or APIs based on standards like CORBA or SOAP.

Today’s typical gaps are the fundamental drivers behind the MRA.  Given the expanse between traditional CAD and CAE tools, modularity necessitates having at least the four patterns/representations above.  Just as a type of shock absorber can be used on many different types of cars, a given SMM type (e.g., an Ansys FEA SMM) can be used by many types of ABBs; a given ABB can be used by many CBAMs; and so on. 

Industrial applications include airframe structural analysis, PWA-B design and thermomechanical analysis, and electronic chip packaging thermal resistance analysis and thermomechanical analysis.  Results include decreasing total simulation cycle time by 75% [1] and leveraging the richness of an ISO 10303 standard product model for electronics: www.ap210.org
2 Preliminary Characterizations

2.1 Analysis Templates, Users, and Computing Environments

With the above as context, this section identifies several XAI metrics and their position within the MRA.

A public Request for Information (RFI) document from the Boeing Common Structures Workstation (CSW) effort [2] identifies industrial needs for analysis templates and their context within complex engineering development environments. 

The RFI is Boeing-specific and aerospace-oriented, and it focuses on the structural analysis domain.  Still, we have found that the abstracted needs and issues occur in other companies and with other product types and analysis domains. 

Documents like this RFI provide useful starting points towards characterizing the magnitude of modeling and simulation (M&S) interoperability problems at a more generic cross-industry level.  

Our multi-representation architecture (MRA) for CAD-CAE integration provides a framework and methodology for decomposing such problems into logical units (based on their engineering meaning and object-oriented thinking).   For example, one can organize and analyze items from the above RFI by identifying which MRA concepts they belong to.  As noted in Section 1 above, Reference [1] points to several studies where we have done such “object-izing” in detail for diverse industrial M&S problems.

The RFI provides the following Boeing statistics either directly or indirectly.  Here we have also identified their MRA context as an initial estimate: 

· 110 groupings of generic structural analysis templates, with each grouping usually containing several actual templates and template elements.  

· Source: Counting leaf items in Appendix B “Required Standard Analysis Methods” in [2a]

· These templates typically represent general engineering analytical concepts and are termed analysis building blocks (ABBs) within the MRA.

· Similar listings exist in structural design manuals from organizations like NASA and the USAF.

· 10,000 product-specific analysis templates

· Source: Section 4.5.1 “BCAG Requirements” in [2a]

· These probably include some of the above ABB-type generic templates, but typically they are product-specific usages of such ABBs (e.g., see “Figure 1-3 Part-based Analysis Template” in [2a] for a specific structural part).  These are called context-based analysis models (CBAMs) or simply “analysis templates” in the MRA.

· 1800 users (for structural analysis-oriented capabilities) located “at many sites, world wide”

· Source: [2b] 

· These engineer users typically create CBAM-like models that idealize their specific designs and apply generic analysis methods.  Their designer counterparts (i.e. CAD users focused on manufacturability and other detailed requirements) typically would like to use pre-developed CBAM-like templates.

· Example variety and number of hardware/OS platforms, software tools, and related technologies.  From another domain and organization (Figure 2), one can count ~45 major engineering tools.

· Source: [2b] and Figure 2 below from JPL.

· Development organizations typically maintain detailed listings of the CAD/E software they use (some organizations use O(100) such tools; imagine the configuration/version control issues this causes, as each tool will typically evolve with a new release every 1-2 years).

· CAD models and parts libraries are related to APMs in the MRA.

· CAE models (for engineering analysis like FEA) and associated libraries (for material behavior, loads/environments, etc.) are related to SMMs in the MRA.

[image: image4.wmf]Holding Account

Billing & Payable

E

-

CAE Toolsmiths and Workstations

DNP Operations

Toolsmiths

Workstations

System

System

CAE

CAE

DIVISION 31

DIVISION 31

System

System

CAE

CAE

DIVISION 31

DIVISION 31

Servers & Sys Admin

M

-

CAE Servers

& Sys Admin

Servers & SA

TMOD Severs

* Not DNP Operations

Design, Build, Assembly, Test (

DBAT

) Process

Design, Build, Assembly, Test (

DBAT

) Process

CAE Cost Centers

JPL Projects and Technical Divisions

Customers

Robin 

Robin 

Moncada

Moncada

Management and Administration

Robin 

Robin 

Moncada

Moncada

Management and Administration

Soap

Sat Took Kit SDK

Doors

ApGen

Fast Flight

Ansoft 

HPEE 

Sof

Sonnet

Mentor Graphics

Cadence

Mathworks

Matlab

Synopsys

Synplicity

Ilogix

Statemate

Orcad 

AutoCad 

Relex

Avant

!

Place & Route

-

Actel

-

Xilinx 

-

Atmel

(and many more)

PTC Computer Vision

PTC Pro

-

E

SDRC Ideas

SDRC 

Femap

Solid Works

Cosmos

NASTRAN

Adams

Sinda

/Fluent

PDMS 

-

EDMG

SDRC Metaphase

Sherpa

Visual 

ToolSets

Cool 

Jex

Perceps

Rational Rose

Ruify

Harlequin LISP

I

-

Logix 

Rhapsody

Code V

LensView

TracePro

Zemax

Software Tools

Electronics

Electronics

CAE

CAE

DIVISION 34

DIVISION 34

Electronics

Electronics

CAE

CAE

DIVISION 34

DIVISION 34

RF & EM

RF & EM

CAE

CAE

DIVISION 33

DIVISION 33

RF & EM

RF & EM

CAE

CAE

DIVISION 33

DIVISION 33

Mechanical

Mechanical

CAE

CAE

DIVISION 35

DIVISION 35

Mechanical

Mechanical

CAE

CAE

DIVISION 35

DIVISION 35

Software

Software

CAE

CAE

DIVISION 36

DIVISION 36

Software

Software

CAE

CAE

DIVISION 36

DIVISION 36

Optical

Optical

CAE

CAE

DIVISION 38

DIVISION 38

Optical

Optical

CAE

CAE

DIVISION 38

DIVISION 38

M

-

CAE Toolsmiths

and Workstations

Adapted from “Computer Aided Engineering Tool Service at JPL” 

-

2001

-

07

-

22 

-

Mike Dickerson 

-

NASA

-

JPL


Figure 2 - Major engineering tools in the NASA JPL Design Hub [3]

2.2 Identifying CAD-CAE Associativity Gaps

The upper left portion of Figure 3 exemplifies one of the root problems faced in modeling and simulation today: the fine-grained associativity gap [1, 4].  On the left is a detailed design model (a CAD model focusing on shape and assembly) and on the right is one analysis model for one feature in this assembly (a formula-based CAE model for estimating fitting feature strength).   This CAE model has some 20 idealized parameters related to shape, material, and loading.  Each idealized parameter is related to one or more detailed parameters in the CAD model.  For example, Figure 4 illustrates two shape-oriented relations for this example (for idealized parameters b and te). The MRA organizes such relations inside APMs and CBAMs to maximize their modularity and reusability (a recent project report [4] overviews the MRA implementation of this example).

Today associativity between such models is typically represented in a computer-sensible form only at the macro level, if at all.  For example, a product data management (PDM) system might tell you “part P1 has an analysis model A1” (a macro-level associativity relation), but it contains nothing about detailed relations like in Figure 4 that exist between attributes inside P1 and A1.  

These root-level relations are where CAD-CAE interoperability actually occurs.  Due such gaps, engineers must expend manual effort enforcing and maintaining these associativity relations.  

2.3 Costs of CAD-CAE Associativity Gaps

Having identified the fine-grained associativity gap, one can begin to estimate related costs.  First the categories of problems this gap causes are identified in the upper right portion of Figure 3.  Next, we focus on one such problem in the calculation in this figure: the manual labor cost to maintain associativity between CAD models and structural CAE models in a single complex system like a commercial airframe.  Table 1 explains each of the attributes employed in this order of magnitude calculation.

Table 1 - Attributes used in the Figure 3 associativity cost estimate.

	Attribute
	Explanation for order of magnitude values used

	Number of “analysis-significant” parts per complex product.
	10,000 is a rough estimate.  The Boeing 777 has some 3 million total parts [5], so this number is less than 1% of the total parts in that case.  


Here “analysis-significant” indicates a part that needs some type of physics-based analysis to ensure that it meets specified requirements (not all parts have such needs, e.g., if they do not experience significant loading).  

	Number of analysis templates per part.
	10 is an estimate based on experience with a variety of such templates.  These templates are typically categorized as CBAMs per the MRA.

	Number of variables (and related gaps) per analysis template. 
	10 is again an estimate based on experience with templates like that in Figure 4.  Such variables are typically idealized values related to shape, material, and boundary conditions (e.g., loads).  Each one is usually involved in a distinct associativity relation in the CBAM. 


Each variable represents a gap in that a user must transfer the value from a design model or other source and place it into the analysis template, and then do this each time the original source changes.

	Cost per gap 
	$10 is a (conservative?) estimate assuming that each variable takes on average a few minutes to find and manually transfer into the analysis model of interest.  Some such transfers are simple equality relations, whereas others involve computing an analysis model attribute from one or more design model attributes (as for parameters b and te in Figure 4).  This estimate does not consider multiple transfers or reverse transfers (from CAE to CAD).


Note that this simple cost model deals only with one category of associativity gap costs.  Other cost categories, higher fidelity models, and considering other gaps, will help better estimate the true costs of the CAD-CAE interoperability problem.  For example, statistics for part type and part occurrence-specific analysis would help determine a more accurate estimate for the number of “analysis-significant” parts.  Also some organizations have developed in-house codes to automate associativity for some analysis templates.  Thus, more detailed cost estimates should consider these cases and the costs necessary to develop such aids. 

CAD systems are making progress in capturing parametric relations (e.g., CATIA v5). However, challenges remain in terms of knowledge modularity, reusability, and accessibility (e.g., the relations are often buried inside compiled code or proprietary vendor formats), not to mention directionality, fidelity, control, and multi-disciplinary associativity.  Facilities for efficiently defining and managing template-level associativity relations is becoming a significant meta-issue (beyond managing the specific values the relations produce for a given design-analysis instance).

[image: image5.wmf]Categories of Gap Costs

•

Associativity time & labor

-

Manual maintenance

-

Little re

-

use

-

Lost knowledge

•

Inconsistencies

•

Limited analysis usage

-

Fewer parts analyzed

-

Fewer iterations per part

•

“Wrong” values 

-

Too conservative: 

Extra part costs and

performance inefficiencies

-

Too loose: 

Re

-

work, failures, law suits

(

)

(

)

(

)

(

)

(

)

(

)

(

)

000

,

000

,

10

$

gap

$

10

 

gaps

000

,

000

,

1

gaps

000

,

000

,

1

analysis

variables

 

10

part

analyses

 

10

parts

 

000

,

10

O

O

O

O

O

O

O

=

´

=

´

´

e

se

t

r

P

f

0

2

p

=

2

1

e

be

ht

P

C

f

=

)

,

,

(

1

3

h

b

r

f

K

=

Analysis Model

(with Idealized Features)

Detailed Design Model

Channel Fitting Analysis

idealizations

G

No explicit

fine-grained

CAD-CAE

associativity

Initial Cost Estimate per Complex Product

(only for manual maintenance costs of structural analysis proble

ms)


Figure 3 - Costs of CAD-CAE associativity gaps

[image: image6.wmf]Detailed

Features/Parameters

Tagged in CAD Model 

(CATIA)

z

f

x

f

cavity3.base.minimum_thickness

y

f

x

f

rib8

cavity 3

rib9

= t

8

,t 

9

rib8.thickness

rib9.thickness

cavity3.width, w

3

z

f

y

f

x

f

z

f

x

f

y

f

G

i

-

Relations between 

idealized CAE parameters

and 

detailed CAD parameters

G

1

: 

b

= 

cavity3.inner_width + rib8.thickness/2 + rib9.thickness/2

G

2

: 

t

e

= 

cavity3.base.minimum_thickness

Idealized Features

in CAE Model

Tension Fitting Analysis

y

f

Often missing in 

today’s process

G

2

G

1

t

e

b


Figure 4 - Example fine-grained CAD-CAE associativity relations [4]

3 Summary

This report presents a preliminary study to more precisely characterize X-analysis integration (XAI) issues like CAD-CAE interoperability. It identifies several engineering information metrics and example values from the aerospace industry: number of structural analysis users, number of analysis templates, number of computing environment components (e.g., number of CAD and CAE tools used in an example aerospace electronics design environment), and number of CAD-CAE associativity relations. 

We use a combined top-down and bottom-up approach in this study. The multi-representation architecture (MRA) guides top-down problem decomposition.  It also identifies several abstraction patterns that exist in diverse CAD and CAE environments.  From this conceptual framework we can narrow in and define metrics for each type of pattern.  

The bottom-up facet involves recognizing basic issues that occur within several patterns at the root level.  In this report we describe fine-grained associativity as one basic issue: how to connect attributes within and between diverse models.  An example analysis template is given containing some 20 CAD-CAE associativity relations. Each relation represents an associativity gap in typical practice today where a person must manually enforce such relations (i.e., transform data from one model into another model). From experience with such examples we provide an estimate that some 1 million gaps exist in the structural analysis of a complex product like an airframe.  The gap-related costs likely run into the tens of millions of dollars.

Thus we identify supporting fine-grained associativity as a fundamental need.  This bottom-level capability, from which higher-level capabilities are built, is analogous to the transistor and other circuit elements in electronics. The transistor’s seemingly simplistic ability to provide 1’s and 0’s is the foundation for digital logic on which the information age has been built.  Top-down architectures like the MRA have similar electronics analogies.

Constrained object (COB) technology is one step towards enabling generalized fine-grained associativity. It has basic information processing elements analogous to electronic elements like the transistor.  Constraint schematics, analogous to electrical schematics, combine COB elements to form higher level objects like analysis templates.  These templates are in fact instances of MRA patterns; hence, bottom-up associativity capabilities are joined to top-down M&S integration concepts.

The above cost estimate helps one think about the problem in more precise terms.  We recommend further studies to refine cost estimates and target fundamental issues and solutions.  For example, the Engineering Framework Interest Group [6] is addressing other basic gaps with similar potential impact (i.e., the content coverage gap and the content semantic gap).  Similar examples from other domains are available to characterize broader interoperability problems and guide solution approaches like the MRA and COBs [1].  

Acknowledgments

Special thanks to Richard Neal and the folks at IMTI for motivating this paper with their roadmapping efforts [7].  

References

[1] 
R. S. Peak (March 15, 2000) X-Analysis Integration (XAI) Technology, Georgia Tech EIS Lab Technical Report EL002-2000A, http://eislab.gatech.edu/.

Overviews Georgia Tech analysis integration research and applications to enable enhanced simulation-based design and engineering.  Includes an annotated bibliography.

[2] Request for Information (RFI): Common Structures Workstation (CSW). (June 14, 2000).  The Boeing Company.  Available at http://eislab.gatech.edu/projects/boeing-psi/2000-06-csw-rfi/ - includes:

[2a] Attachment A - “Common Structures Workstation - Design Requirements and Objectives”

[2b] Attachment B - “Boeing Computing Infrastructure”

[3] 
R. Peak, M. Dickerson, L. Klein, S. Waterbury, G. Smith, T. Thurman, J. U'Ren, K. Buchanan 
(April 9-12, 2002) Progress on Standards-Based Engineering Frameworks that include STEP AP210 (Avionics), PDM Schema, and AP233 (Systems): An Engineering Framework Interest Group (EFWIG) Overview. 2002 NASA-ESA Workshop on Aerospace Product Data Exchange, The Netherlands.

[4] 
R. S. Peak, R. E. Fulton, A. Chandrasekhar, S. Cimtalay, M. A. Hale, D. Koo, L. Ma, A. J. Scholand, 
D. R. Tamburini, M. W. Wilson (Feb. 2, 1999) Design-Analysis Associativity Technology for PSI, Phase I Report: Pilot Demonstration of STEP-based Stress Templates, Georgia Tech EIS Lab Technical Report 
E-15-647-D1, The Boeing Company Contract W309702, http://eislab.gatech.edu/projects/.
[5] 
Boeing 777 Facts. http://www.boeing.com/commercial/777family/pf/pf_facts.html as of 2002-08.

[6]
For further information, see resources including the following:

· http://eislab.gatech.edu/research/XAI_Central.doc
· Recent project work. http://eislab.gatech.edu/projects/
· Engineering Framework Interest Group (EFWIG). http://eislab.gatech.edu/efwig/
[7]
"Modeling and Simulation for Affordable Manufacturing" Information Center at http://www.imti21.org/  (as of 2002-08).










� 	X = design, mfg., sustainment, and other lifecycle phases.





For general case (may depend on sponsor)
Copyright © 2001 by Georgia Tech Research Corporation, Atlanta, Georgia 30332-0415 USA.  All Rights Reserved. 
Permission to distribute without changes and at no charge is hereby granted provided this notice is included.

If XaiTools or other software:

Copyright © 2001 by Georgia Tech Research Corporation, Atlanta, Georgia 30332-0415 USA.  All Rights Reserved.  XaiTools, XaiTools ChipPackage, XaiTools PWA-B, U-Engineer, and U-Engineer.com are trademarks of GTRC.  Developed by the Engineering Information Systems Lab, Georgia Institute of Technology, http://eislab.gatech.edu/tools/XaiTools/ - Warning: This computer program, including documentation and data files, is protected by copyright law and international treaties.  Use, reproduction, and/or distribution of this program is prohibited except as authorized by a written license agreement.

22
Copyright © 2002 by Georgia Tech Research Corporation, Atlanta, Georgia 30332-0415 USA.  All Rights Reserved.  Permission to reproduce for non-profit purposes (including internal corporate usage) is hereby granted provided this notice and a proper citation are included.


_1012389126.bin

